“WiMax the Future of Broadband Internet Access”

Written By: Mike Korenbaum

CSU 645: Network Security

Professor: Ravi Sundaram

December 11, 2005

1.0 Introduction

During the mid 1990’s to the present, personal computing in conjunction with the development of the World Wide Web has put an increased emphasis on the use of computers and computer technology in today’s society.  One major facet of the computer technology field that has had tremendous impact on the way we use computers today, especially in the corporate world, is computer networking.   Networking technologies have transformed over the years, and vastly improved to accommodate the demand for high bandwidth with maximum mobility.  Until recently, the standard in networking has been to implement a wired local area network (LAN) or metropolitan area network (MAN), the most common implementation Ethernet (802.3).  In 1999, a new networking standard emerged (802.11b) that would revolutionize the way computer networks are implemented.  With the establishment of the 802.11b standard in 1999, IEEE modified Ethernet networking with the addition of new physical and data-link layers to the ISO model, thereby providing Ethernet over a radio frequency (RF) [1].

About five years later wireless technology has had time to work through many cycles of development and testing of hardware, software, and security.  “Wireless networking applications continue to proliferate at an incredible pace as wireless features, functions, security, and throughput improve” [1].  Products which employ the use of wireless technology have a broad range of uses from enterprise to home users.  In most networking situations, whether wired or wireless, the company and end users require access to the internet.  The increase in broadband internet access providers (i.e. DSL or Cable), has met that demand in terms of providing the backhaul for WiFi hotspots, home networks, and small business networks.  “Results of three independent studies showed that notebook users averaged around hours of new productivity per week when they received wireless networking capability” [2].  It is quite evident, with the increase in mobile computer users, constant access to the internet proves to be a vital asset.

The current trend, especially in metropolitan areas, is the implementations of WiFi hotspots, which are fed by fixed broadband connections.  While hotspots provide a 100 – 300 meter solution, WiMAX has the ability to provide the “last mile” solution.  Users of a WiMAX based connection would not require a fixed broadband connection, thus increasing mobility.  One can see this technology has the ability to provide broadband internet access to sections of metropolitan areas, as well as rural or remote areas in which broadband internet access is not available.

“WiMAX is all about delivering broadband wireless access to the masses” [2].  It is an inexpensive alternative to DSL, and cable broadband access.  Costs for installation of a wireless infrastructure are far less than today’s wired solutions, which require laying cables and ripping up streets.  WiMAX or Worldwide Interoperability for Microwave Access is a certification mark for products that conform to the standards of the IEEE 802.16 working group.  Technologies created under this certification are aimed at providing wireless broadband internet access.  The IEEE 802.16 group, provide standards to support wireless metropolitan area networks (MANs), unlike the IEEE 802.11a/b/g standards, which were written to support LANs.  Wireless equipment providers have extended the capability of 802.11 technologies by writing proprietary media access controllers (MAC) to support specific physical layers (PHY) optimized for outdoor citywide delivery of wireless broadband. The problem, of course, was the technologies created by different vendors were proprietary, and thus suffer compatibility issues.  Organizations, such as the WiMAX Forum, are ensuring that newly developed wireless broadband technology conforms to the specifications put forth by IEEE 802.16.  Wireless broadband technology in conjunction with strict standardization has the potential to revolutionize the way in which we will access the internet in the future.

2.0 Technical Information/ Existing work

2.1 WiMax Competitors

Within the wireless world there exist many technologies looking to provide the next generation of broadband internet access, multimedia delivery, data transmission and telephony services.  Proponents of these technologies are cellular carriers, proprietary wireless providers, chipset vendors, the Institute of Electrical and Electronics Engineers, and many more.  Currently there are various next generation wireless technologies, implemented via chipsets, radio transceivers, and antennas.  The following table outlines some of these wireless technologies, their capabilities, and successful deployments or current status.

| Wireless Technology | Capabilities | Deployments |
| --- | --- | --- |
| 2.5G enhanced data GSM environment (EDGE) | Average download of 135Kbps. | Cingular’s current data network. Approximate cost $35.00/mo. |
| Evolution Data Optimized (EV-DO) | Average throughput of 300 – 512Kbps Requires special PCMCIA card approx $80 each. | Verizon and Sprint currently deploy EVDO networks in major cities across the U.S. Approximate cost $80.00/mo. |
| 3.5G high speed downlink packet access (HSDPA) | Projected download of 14.4Mbps, but actual downstream speeds will be 1Mbps. Primarily supports applications that require one-way communication because of slow uplink speeds. | The following carriers have started trial HSDPA networks: UK’s Orange, O2, and Vodafone; Japan’s NTT DoCoMo; and Cingular. |
| Wireless Broadband (WiBro) | Korea’s version of mobile WiMax (802.16e). It will incorporate 802.16d and 802.16e standards. The standard will allow for 1Mbps to mobile users (i.e. subscribers moving at 60 to 70 km/h). | Korea's Ministry of Information and Communication (MIC) approved the IEEE 802.16 standard as Korea's base technology for portable wireless broadband access and set aside spectrum for Wi-Bro in the 2.3 GHz band. Predicted to take off in 2006 and attract 9.1 million subscribers. |
| IEEE 802.20 | Aims to provide greater than 1Mbps per user. Utilizes licensed bands below 3.5 GHz. Constant connectivity for users in MAN environment traveling up to 250Km/h. | Established by IEEE in December of 2002. This standard is still a work in progress. |
| Fixed WiMax (IEEE 802.16d) | Created to provide “last mile” solution to users within a MAN environment. Each base station will provide a theoretical 50Km range of transmission (typical setups will most likely use a 5 – 10Km range). 802.16d can provide data rates of 75Mbps on a single channel (both up and downlink). | WiMax equipment is currently being tested and put into trial networks. CPU manufacturers such as Intel and Fujitsu have developed fixed and mobile WiMax chips. Airspan, Aperto and Alvarion will utilize Intel chips for WiMax gear. AirZed has a WiMax network deployed in Malaysia offering 1Mbps Down and 384Kbps up per subscriber. |
| Proprietary wireless broadband networks | Companies utilize 802.11 mesh networks or write proprietary media access controllers to support specific physical layers to provide outdoor broadband internet access in metropolitan areas. | Companies such as Clearwire offer wireless access in selective cities. Most of Clearwire’s coverage is in California |
| Mobile WiMax (IEEE 802.16e) | The next step for WiMax, which will provide mobile wireless broadband access to subscribers. It is scheduled to have a 1Mbps data rate or higher for users traveling around 50 – 60Km/h. | IEEE is set to complete the standard sometime in 2006. Networking equipment manufacturers are already developing and testing pre-802.16e products. |

[9] Table 1

2.2 Overview of WiMax

WiMax is a broadband wireless access (BWA) technology that is aimed at providing wireless MANs.  Unlike its predecessor WiFi, which has a maximum broadcast radius of approximately 300 meters, WiMax technology is predicted to have a range of up to 50 kilometers.  Such a wireless technology will be able to cover an entire metropolitan area with a small number of base stations.  A similar concept to that of WiFi networks, in which wireless access points (APs) provide connectivity to the network, WiMax base stations utilize cell towers already in place as a means of transmission.  “A WiMax antenna can “share” a cell tower without compromising the functionality of cellular arrays already in place” [3].  In order to provide internet connectivity the WiMax base station will connect to a switching facility via a hard wire link or some sort of line of sight backhaul (i.e. Free Space Optics or a high powered RF system).  In turn the switching facility would connect to either an ISP to switch traffic into the internet or the data is switched into the public telephone system.  Figure 1 depicts how a typical WiMax network would be set up.  Predicted data rates for WiMax networks are approximately 75 megabits per second (Mbps) per base station.  According to Intel, if typical cell sizes are 2 to 10 kilometers, at 75 Mbps this is enough bandwidth for each base station to simultaneously support more than 60 businesses with T1/E1- type connectivity and hundreds of homes with DSL-type connectivity [2].  With the ability to transmit at 75Mbps per channel, according to Edward Rerisi the director of research for Allied Business Intelligence, it is recommended that each base station provide connectivity within a 10 mile radius and to approximately 500 subscribers at DSL-type or better connectivity [7].  A subscriber will typically have an external or internal antenna (the subscriber station) and will connect to the closest WiMax base station.

[8] Figure 1

2.3 IEEE Standards

The 802.16 working group was charged with defining an air interface standard for BWA systems, based on a point-to-multipoint wireless infrastructures, which operate at radio frequencies between 10 GHz and 66 GHz [2].  The original specification was chartered by the 802 working group in March of 1999.  Some of the major goals of this newly formed working group were: to use wireless links with microwave or millimeter wave radios, use the licensed spectrum (11 GHz or higher), work on the scale of metropolitan areas, use a point to multipoint architecture, provide quality of service, and provide an overall throughput greater than 2 Mbps.  The original specification applied only to line-of-sight deployments in the licensed spectrum.  Later revisions of 802.16, specifically 802.16a, took into account the 2 GHz – 11 GHz frequency spectrum, which allowed for support of non-line-of-sight (NLOS) deployments within both the licensed (2.5 -2.69 and 3.4 – 3.6 GHz) and the unlicensed spectrum (5.725 – 5.850 GHz) [4].  Also within the 802.16a revision, three new PHY-layer specifications were defined:  single-carrier layer, a 256-point fast Fourier transform orthogonal frequency division multiplexing (FFT OFDM), and a 2048-point FFT OFDMA layer [2].  The 256-point waveform is utilized by both WiMax and HiperMAN (the European equivalent of WiMax), to ensure worldwide interoperability.  Because of the great deal of options within the original standard, IEEE 802.16c is an update made to detail system profiling; outlining what are mandatory and optional features.  As of 2004 IEEE 802.16-2004(d) has been rolled out to encompass the updates of the previous 3 802.16 standards.  It also has outlined support to reduce MAC overhead, and support for multiple polling methodologies.  The 802.16d standard has also incorporated the use of MIMO (Multiple-input-multiple-output) antennas to strengthen and enhance the level of transmission by base stations.  A standard still in the developmental stages is 802.16e, which conserves the basis of the fixed WiMax standards outlined above, while adding support for mobile broadband.  Preliminary findings suggest the mobile feature will be based upon the OFDMA.  OFDMA (sometime referred to as Multiuser-OFDM) is an extension of OFDM, which is currently the modulation choice for 802.16-2004(d).  The requirement for non-line-of-sight capabilities requires consideration for which physical layer to utilize when implementing WiMax technology.

2.4 PHY

There are four physical layers defined by the 802.16 specification.  However, only three are applicable to the 2 – 11 GHz spectrum, which allows for NLOS capabilities.  The three different PHY’s that can be use in combination with the MAC layer to provide a reliable end-to-end link are:  WirelessMAN-SCa, WirelessMAN-OFDM, and WirelessMAN-OFDMA.  WirelessMAN-SCa is a single-carrier modulated air interface. The other two physical specifications are based on Orthogonol Frequency Division Multiplexing and are more suitable for non-LOS operation due to the simplicity of the equalization process for multicarrier signals [4].  In OFDM, a single user is able to transmit on all of the sub-carriers at any give time.  In order to support multiple users a frequency division duplexing (FDD) or time division duplexing (TDD) scheme is utilized.  FDD allows for full or half duplex connections from the subscriber station to the base station.  OFDMA will allow multiple users to transmit simultaneously on the different sub-carriers by employing complex algorithms to ensure successful transmission [5].  Current WiMax technology makes use of 256-FFT OFDM, which allows the frequency band to broken into 256 signal carriers.  The data can then be distributed across multiple carriers simultaneously, and combined into channels.  This is similar to the PHY of 802.11 networks, however in 802.16d the channels have the ability to grow or shrink according to the required throughput for particular channels.

This scheme seems to be favored by the vendor community because of lower peak to average ratio, faster FFT calculations, and more flexible requirements for frequency synchronization compared to the 2048-OFDMA carrier.  Of the 256 subcarriers defined 192 are used for user data, 56 are nulled for a guard band, and eight are used as permanent pilot symbols [4].

The IEEE 802.16 standard has also specified variable channel bandwidths, of which the WiMax forum is trying to decide is the best solution to provide interoperability between vendors implementing the 802.16 standard.  Currently channel bandwidths can be any multiple of 1.25 MHz, 1.5 MHz, and 1.75 MHz, with a maximum of 20 MHz.  This provides flexibility to operate in many different frequency bands with varying channel requirements around the world.

2.5 Access and Security

2.5.1 MAC

Similar to 802.11, the 802.16 media access control (MAC) protocol, governs the clients access to the physical medium.  The MAC was designed for point-to-multipoint broadband wireless access applications.  The scheduling algorithm within 802.16 MAC protocol offers optimal prioritization of traffic based on a FIFO system.  Clients are assigned bandwidth based on time of initial access, instead of random queue assignment based on MAC address as in 802.11.  The MAC layer protocol has the ability to meet very high data rate applications with a variety of QoS requirements.  The MAC is flexible and efficient over a vast range of data traffic models; allowing 802.16 deployments to support hundreds of terminals per channel and have multiple end users share each terminal.  Another consideration is 802.16 balances clients’ needs, instead of a best effort delivery system, to ensure quality of service.  The system has been designed to support legacy time division multiplexing voice and data, IP connectivity, and voice over IP [4].  The MAC layer is connection oriented and has two types of connections, management and transport.  Management connections, which are unencrypted, handle bandwidth requests, initial ranging between the subscriber station (SS) and base station (BS), broadcast data, and general management messaging.  Transport connections, which are encrypted, are where the SS is authorized to BS, and data is encrypted and sent between the two.

2.5.2 Network Entry

The following describes how a subscriber station accesses the WiMax network provided by a base station.

- SS scans for BS downlink signal and establishes channel parameters.

- Initial ranging and PHY parameters are determined between the SS and the BS.

- The Privacy key Management (PKM) protocol authorizes the SS to the BS.

- The SS registers with the BS by sending a request message to the BS.  The BS’s response assigns a connection ID for a secondary management connection.

- The SS and the BS create transport connections [10].

In the following section the security algorithms implemented in 802.16d will be described in detail.

2.5.3 Security

With regards to security, early implementations of wireless (802.11) networks utilize WEP or WPA security protocols to encrypt the radio signal, which have been and are easy to crack.  On the contrary, 802.16 by default employ a baseline authentication architecture based on PKI (public key infrastructure) certificate authorization.  In this system the base station validates the client’s digital certificate before permitting access to the physical layer [6].  802.16 security is implemented as a privacy sub-layer at the bottom of the MAC protocol’s internal layering.  The security architecture contains five main elements: security associations (SAs), X.509 certificates, Encryption, Privacy and Key Management (PKM) authorization, and the PKM protocol.  802.16 uses two types of SAs, an authorization SA and a data SA.  The authorization SA contains the X.509 certificate, an authorization key (AK), key encryption key (KEK), and both an up and downlink hash-function based message authentication code (HMAC).  The AK acts like the master key in the SSL/TLS encryption scheme.  The KEK is a 112-bit 3-DES key used to distribute the TEKs.  The HMAC is a hash function of the AK used to show the BS that a particular SS knows the proper AK for a particular connection.  The X.509 certificate is generated by the manufacturer of the SS equipment (i.e. the manufacturer is the CA).  The data SA consists of a SA identifier (SAID), two traffic encryption keys (TEKs), and an Initialization Vector (IV) to determine the next TEK when the original expires.  The TEK is used to encrypt the data sent between the SS and BS once authorization is completed.  The TEK expires within a specified time period (between 30 minutes and 7 days), hence the BS maintains the IV which determines the next TEK to use.  The PKM authorization protocol (illustrated by figure 2) defines how the SS is authorized to the BS utilizing the Authorization SA; this is accomplished via a 3-way handshake initialized by the SS.  The subscriber station sends in two messages, the certificate of its manufacturer, its certificate, its capabilities, and the SAID.  The BS uses either the SS certificate or both the Manufacturer certificate and the SS certificate to validate the SS as an authorized users.  The BS then uses the SS’s public key to encrypt the AK sent in the third message along with the lifetime of the AK, a sequence number, and a SAIDList that defines parameters on how the SS and BS will communicate.  The privacy and key management protocol is used to establish a data SA between the SS and BS.  The PKM protocol consists of a two or three way handshake, which is dependent upon whether the BS forces re-keying of the TEK (illustrated by figure 3).  The first two messages contain an HMAC of the AK which allows both the SS and BS to detect forgeries respectively.  The third message, which is sent by the BS, gives the SS the new TEK (IV + AK = new TEK) to encrypt subsequent messages.  Although the encryption scheme of 802.16 seems secure because of its use of public key cryptography to distribute the secret keys (AK, KEK, TEK) there are still vulnerabilities within the system that can be exploited by an attacker.

Figure 2

BS

Cert (Manufacturer(SS))

Cert(SS) | Capabilities | SAID

RSA-Encrypt(Pub-key(SS), AK) | Lifetime | Seq No | SAIDList

SS

Figure 3

BS

SeqNo | SAID | HMAC (1)

SeqNo | SAID | HMAC (2)

SeqNo | SAID | OldTEK | NewTEK| HMAC(3)

SS

3.0 Problem & Proposed Solution

3.1 Issues with Security Defined by 802.16

Since WiMax is a wireless technology it is prone to PHY-level attacks as well as MAC level attacks.  A typical PHY level threat arises when an attacker sends multiple frames to a BS and essentially performs a denial of service on the physical medium that BS is transmitting on, as well as effectively draining the battery of the receiver.  Another security flaw inherent in any RF technology is the possibility some one may jam the radio spectrum, and in turn denying service to all parties.  However, in this paper I will focus only on the MAC-level threats.

Some of the MAC-level threats exist only because there is a lack of explicit definition in the 802.16 standard.  One major issue is that the specification never defines a method for the base station to identify itself to the subscriber station, thereby allowing an attacker to spoof a base station and comprise a subscriber’s network or sensitive data.  Another issue arises in the way the AK is generated; the AK is generated solely by the BS, thus the SS must implicitly trust that the BS has generated a safe unique key.  Since the AK is an integral part of the encryption scheme it needs to be cryptographically unique from any other BS’s AK.  Also another issue that arises with the BS is the fact that there is no distinction between authorization SAs on the BS.  This security flaw leaves the protocol suite open to replay attacks [10].  Since there is no distinction between authorization SAs there is also no distinction between data SAs, thus the encryption scheme is vulnerable to key reuse.  Because the 802.16 specification defines the use of a DES CBC to encrypt the MPDU (MAC protocol data units) this leaves the network open to replay attacks and an attacker can forge or modify frames from authorized users and inject them back into the network.  The following section will outline a list of changes that will safeguard against the MAC level threats listed above.

3.2 Proposed solutions to MAC level threats

To defend against replay attacks and verify data authenticity a new encryption scheme should be utilized to encrypt the MPDUs.  A suggestion would be to use the Advanced Encryption Standard (AES) with cipher block chaining MAC (CCM) for data encryption [10].  AES requires that the transmitter construct a unique nonce per packet, thus enabling data to be secured and prevent replay attacks.  “The IEEE 802.16e working group has recently adopted AES in CCM mode as a new data link cipher” [10].  The other main issue was lack of identification of the BS to the SS.  To solve this issue add a BS certificate to the third message of the PKM authorization protocol.  By having the BS send its certificate and signature to the SS, this eliminates the possibility of having a spoofed BS (provided that the SS equipment has an up to date list of BS certificates).  The issue of not having unique authorization SAs and not knowing if the AKs were unique either can be addressed by modifying PKM authorization protocol as well.  If both the SS and BS each add their own generated random numbers and put them into the handshake, then the BS will generate the AK utilizing the random numbers, thus reassuring the SS that the BS has generated are fresh.  Also the BS will utilize its MAC address and the SS’s MAC address to generate the AK, thereby binding this AK to only these two peers (i.e. the AK is unique to only 1 BS and 1 SS)[10].

3.3 Other security considerations

A successful implementation of an 802.16 based network will require more than just the PKI certificate authorization required to access the physical layer.  Subscribers to WiMax services will have to implement security measures to protect the end users.  Software based threat management and secure access systems will be vital to preserve the integrity of such networks.  Infrastructures will need to implement components such as virtual private networks (VPNs), firewalls, and intrusion prevention systems, each residing at the Application layer.  A call to the manufactures of WiMax compliant technology to implement these types of security measures will be require for the success and proliferation of 802.16.  Companies such as Hifn and Intel have developed processors to enable developers and OEMs of WiMax technologies to create products which keep in mind these security issues.  Hifn has developed the 7955/7956 security processors designed for networking applications like VPN broadband routers, wireless access points, firewall/VPN appliances, and other CPE.  These security engines accelerate security protocols such as DES, 3DES, etc.  Intel’s Pro/Wireless 5116 chip will lead the way to more secure, proficient WiMax products.  It has the ability to support in-line security processing using advanced encryption (i.e. 3DES).

4.0 Conclusion

WiMax technology has the ability to be the next generation in broadband internet access.  By the pure fact that it is a wireless technology means that there are less initial installation costs than that of typical DSL or cable services.  “According to Greg Caltabiano, Soma’s chief operating officer; a 200-square-kilometer service area costs a DSL provider over $11 million. The same area can be served wirelessly for about $450,000” [11].  This is a huge advantage for potential WiMax providers, especially in areas around the world where typical DSL or cable can not be implemented because of expensive installation costs.  Fixed WiMax can also provide the backhaul to WiFi networks, thereby creating a hybrid wireless network.  Handoffs between WiMax and WiFi networks will have to be addressed, but I’m confident there will eventually be dual-mode PCMCIA network cards for these types of networks.

The one detail that may slow down the take off of WiMax networks is the band in which the FCC determines they can transmit in.  Basically it is going to be a bidding war between the big Telco and ISP companies to consume all of the licensed bands within the 2 – 11 GHz spectrum.  There is always the option to transmit in the unlicensed bands, however these bands (i.e. ISM 2.4GHz) are becoming over crowded and may cause issues in certain areas.  However, this is not an issue for countries such as Korea, and Europe who have already allocated the 1.9 GHz and 2.3 GHz spectrum to broadband wireless access technologies.

All in all WiMax is a great technology that will eventually take off because of the big name companies that are backing the technology such as Intel and Fujitsu.  Once mobile WiMax (IEEE 802.16e) components hit the market, that’s when you will really see a change in the way you connect to the internet, receive multimedia, and talk on your phone.  Current and future cellular based networks such as HSDPA and EV-DO don’t have the capacity to compete with the throughput, QoS and security that mobile WiMax will be able to provide.  Give it about 3 – 5 years and you will be subscribing to a WiMax provider.

References

[1] S. Kapp, “802.11: Leaving the Wire Behind,” in Internet Computing, IEEE vol.

6, no. 1, pp. 82-85, Jan.-Feb. 2002.

[2] The Technology Vision for WiMax

http://www.intel.com/standards/case/case_wimax.htm.

[3] WiMax, http://en.wikipedia.org/wiki/WiMax.

[4] A. Ghosh and D.R. Wolter, “Broadband wireless access with WiMax/802.16: current

performance benchmarks and future potential,” Communications Magazine, IEEE

vol. 43, no. 2, pp 129 – 136, February 2005.

[5] I. Wong, Z. Shen, B. Evans and J. Andrews, “A Low Complexity Algorithm for

Proportional Resource Allocation in OFDMA Systems,” IEEE International Signal

Processing Systems Workshop, pp. 1-6, Oct. 13-15, 2004.

[6] R. Mylavarapu, “Security considerations for WiMax-based converged network,”

August 2005.

[7] S. Vaughan-Nichols, “ Achieving Wireless Broadband with WiMax,” Computer, vol.

37, no. 6, pp 10-13, June 2004.

[8] WiMax World 11/01/2004

http://www.dailywireless.org/modules.php?name=News&file=article&sid=3216.

[9] G. Lawton, “What Lies Ahead for Cellular Technology,” Computer, vol. 38, no. 6, pp

14 -17, June 2005.

[10] D. Johnston and J. Walker, “Overview of IEEE 802.16 Security,” Security &

Privacy Magazine, IEEE, vol. 02, no. 3, pp 40 -48, June 2004.

[11] S.M. Cherry, “The Wireless Last Mile,” Spectrum, IEEE, vol. 40, no. 9, pp. 18 – 22,

September 2003.